Product Specifications

LeeBoy L600 Tack Distributor

The following specifications are based on the L600 Tack Distributor manufactured by LeeBoy.  Because of continuous equipment research and development, we reserve the right to change these specifications.  For more information, questions or assistance preparing specifications, please contact your local LeeBoy sales representative.

- Dimensions (trailer configuration): - Length: 15’ 8” - Width: 6’ 4” - Height: 5’ 9” - Tank: 600 gallons - Trailer Specifications (for trailer configuration): - 6,000 lb axles with a pintle eye - 5,000 lb jackleg - Wired for towing - Tires: ST 225/75D15 - Electric brakes on axles - Safety chains - Emergency breakaway switch with battery backup - Shipping weight: 2,405 lbs - Loaded weight: 7,275, lbs - Dimensions (skid configuration): - Length: 12’ 8” - Width: 4’ 3” – skid frame width - Height: 4’ 9” - Tank: 600 gallons - Tank Dimensions: - Length: 97.875” - Width: 54.125” - Height: 36.25” - Engine: - 4.8 HP (3.6 kw) Gas Engine - 1 gallon fuel tank - Asphalt Pump: - Specially designed Viking 7 GPM pump - Heat System: - One 300,000 BTU propane burner - One starter burner - 40 lbs propane fuel tank - Temperature Gauge: - 50 - 300⁰ F - Adjustable angle faceplate - Flue Design - Five pass flue system - V-shaped profile - Aluminized exhaust stack - Manhole: - 20” diameter - 10” diameter fill hose - 3 psi pressure relief - Wand: - 5’ 2” wand with cold handles - On/off valve - Removable spray tip - Additional Standard Equipment - Waste bucket mount (trailer configuration only) - 1” Rear tank return - 2” tank overflow - Flush tank: 7 gallons tank with suck back valve - Hose and reel: .5” x 40’ heavy-duty recoil hose reel - Clean out: 4” - Sampling valve: 1” - Optional Equipment: - Engines - Gas Engine - 4.8 hp (3.6 kW) electric start - Hatz diesel - 7.2 hp (5.4 kW) electric start - Gas engine - 12hp (9 kw) electric start - Fuel: 1.32 gallons - Heat Systems - Diesel Burner One 300,000 BTU diesel burner (only available with electric start engines) 7 gallon fuel tank Auto ignition diesel burner with outfire - Overnight Heat 120 volt/1500 watt Immersion element - 12’ Spray pattern - Spraybar controls: In-truck wireless remote controls (requires spray bar option) - Spraybar with cab wireless controls - Spraybar lift with cab wireless controls - Spraybar with cab wireless controls and lift with cab wireless controls - Insulated Tank - Surge brakes in lieu of electric brakes - Frame mounted tool box - Wheel chocks - Fire extinguisher: 5lbs - Training, Support and Company - Training and support manuals - Optional on-site operator training by Dealer - Two-year, unlimited hours warranty against any manufacturer defects - Extended warranty available - 50+ years manufacturing experience - ISO 9001 Certified - Manufactured in the USA