Product Specifications

Rosco Maximizer 4B Distributor

The following specifications are based on the Rosco Maximizer 4B Distributor manufactured by LeeBoy.  Because of continuous equipment research and development, we reserve the right to change these specifications.  For more information, questions or assistance preparing specifications, please contact your local LeeBoy Dealer.

- Dimensions - Length: Depends on type of truck purchased and options selected - Width: Depends on type of truck purchased and options selected - Height: Depends on type of truck purchased and options selected - Weight: Depends on type of truck purchased and options selected - Maximum Tank Capacity: - 2,050 gallons to 5,313 gallons* *Gallons listed are maximum tank capacities. Recommended tank capacities may be limited by 1.) Thermal expansion of materials. 2.) Density of materials. 3.) Truck/axle load ratings. For more detail, please contact your local LeeBoy Dealer. - Tank: - Meets all applicable Federal DOT tank regulations - ASTM A569 grade mild steel with minimum 10 gauge shell and 7 gauge flat heads - Flanged reinforced and welded to shell inside and outside of tank - Seams are all electrically welded - Rear suction sump and front return - Thermometers: One 4” dial thermometer One internal pencil thermometer - Insulation: - 2” rock wool with spacers to prevent compression - Clips to secure position of insulation and protected by an aluminum jacket - Surge Plates: - 10 gauge steel with staggered openings to provide adequate tank strength and proper surge control - Manhole: - Weather tight and safety relieving cover with basket-type strainer - Inside splash guards - Steel measuring stick - Tank vent and overflow - Manhole opening: 22” in diameter - Overflow: 3” in diameter - Sump: - At the rear of tank - Cleanout plug and suction line from the pump includes tank cutoff valve - Sump size: 4” in diameter - Heat System: - Two diesel fired burners with electric ignition and heat limit controls - Two, u-type, high temperature flue pipes running the length of the tank - Single exhaust stack is constructed of stainless steel and includes a rain cover, removable if service is needed - Heating flue size: 8” in diameter throughout - Burners: 560,000 BTU per burner, per hour capacity - Asphalt Pump: - Viking, positive displacement pump - Rotary gear type with built-in relief valve for safety - Asphalt pump driven by low speed high torque fixed displacement hydraulic motor and connected by a flexible coupling - Pump: 560 gallons per minute - Optional Oil Heat - Heat exchanger in lower flue tube - Jacketing on emulsion tank sump - Jacketed Viking asphalt pump - Internal spraybar heating - High-temperature insulated flexible metal hoses and hard line - Distributing Lines: - High temperature, flexible metal hoses - Steel pipe with ball and socket joint is not acceptable - Clean Out: - Enviro-flush system allows solvent circulation through asphalt pump, valves, distributing lines and spraybar - Cleanout material returned to enviro-flush tank for later use or disposal - Flush tank: 25 gallons - Enviro-Flush tank: 25 gallons - Spray System: - Nozzles are brass, slotted and valves are non-clogging - Each has a quick-disconnect feature allowing the operator to change spray widths in 4” increments across the entire bar without the use of tools or blank nozzles - Spraybar: 24’ full circulating, extendible spraybar Two 8’ independently controlled extending sections Each spray nozzle automatically turns on when extended and off when retracted Two 4’ fully circulating hydraulically folding wings Spray nozzles operated in 1’ increments All functions operated by the Plus+1 controller in the cab - Nozzles: Spaced on 4” centers on spraybar - Controller: - Plus+1 CAN Bus control and onboard diagnostic system - 7” high-resolution, color graphic display - Controller for automatic application rate control - Automatic valves for cab control of distributor functions - Exclusive enviro-flush system with hydraulic pump speed control - Settings for flow calibration, speed calibration, application rates, spraybar width and spraybar on/off control by section - Capable of storing up to 12 preset application rates for instant retrieval - Selectable readouts include FPM, GPM, total feet sprayed, total gallons sprayed, total square yards sprayed, flow and speed calibration, application rate, tank temperature, hour meter and spraybar width (English and metric units can be selected for the rate controls) - Controller provides accurate applications regardless of changes in speed or spraybar width during operations - Includes radar horn ground speed sensor - Convenient external controls and pump speed display mounted on rear fender - Hydrostatic Power: - Hydrostatic, front live power driven by engine crankshaft - The hydrostatic pump is an infinitely variable displacement axial piston pump - Hydraulic reservoir: 20 gallons with thermometer and sight gauge and level indicator - Additional Standard Equipment: - Hand spray wand with a 25’ rubber hose - Front and rear dial contents gauge - 4” dial thermometer and internal pencil thermometers - ICC clearance lights and reflectors - Rear bumper to protect spraybar - Sample valve - Back-up alarm - Mud flaps - Ladder and platform assembly - Grease gun - Beacon light - Optional Equipment: - Installation for a customer supplied truck chassis - PTO transmission (required if not using front live power) - LPG burner in lieu of diesel burner [includes 48 gallon LP tank] - Diesel burners with electronic ignition, 100% outfire protection and heat limit control - Diesel burners with outfire protection and heat limit control - Electronically actuated emulsion tank hand rails - Washdown system with pump and hose, self-winding hose reel - Loading hose, 3” x 15’ flexible steel with quick couplers - Loading hose, 3” x 15’ rubber with quick couplers - Hose trough for loading hose - LED working lights, qty 4 - Frame mounted toolbox - Extra fender mounted toolbox - Wet storage box - Full width mud flap - Hot oil option - Electric overnight heat, 240V single phase - GPS in lieu of radar horn - Nozzle selection - Back-up/ reverse camera w/ in cab color display - 20 ft Spray Bar - Exhaust Flue Damper Kit - Training, Support and Company - Training and support manuals - Optional on-site operator training by Dealer - Two-year, unlimited hours warranty against any manufacturer defects - Extended warranty available - 55+ years manufacturing experience - ISO 9001 Certified - Sourcewell Contract# 052417-VTL - Manufactured in the USA