Product Specifications

Rosco Maintenance Distributor

The following specifications are based on the Rosco Maintenance Distributor manufactured by LeeBoy.  Because of continuous equipment research and development, we reserve the right to change these specifications.  For more information, questions or assistance preparing specifications, please contact your local LeeBoy Dealer.

*Published Weights are approximate weight and will vary contingent on options.

- Dimensions - Length: Depends on type of truck purchased and options selected - Width: Depends on type of truck purchased and options selected - Height: Depends on type of truck purchased and options selected - Weight: Depends on type of truck purchased and options selected - 300-Gal Tank: - Rear tank - Rear spray bar - Application rate of 0.01 - 0.035 gal/sq yd - Ball float gauge for material - Load through asphalt pump or manhole - 600-Gal Tank: - Front Tank - Front spray bar - Application rate of 0.01 – 0.035 gal/sq yd - Ball float gauge for material - Load through asphalt pump or manhole - Cleanout: - One tank for new cleaning solution on front and rear system - One tank for used cleaning solution for later disposal on front and rear system - Flush system allows cleaning solution circulation through material pumps, valves, distributing lines and spray bars - Manholes: - 20” diameter - 10” diameter fill hose - 3psi pressure relief - Material Pumps: - Viking, positive displacement pumps (one for each tank) - Rotary gear type with built-in relief valve - Asphalt pumps driven by low-speed high torque fixed displacement hydraulic motors - Spray System: - Power spray bar lift, side shift, and folding wings - Both bars to move together for these functions. - Dual spray bar control to activate/deactivate corresponding nozzles on front and rear bar. - Individual nozzle control allowing the operator to change spray widths in 12” increments across the entire bar from the in-cab controller. - Spray bars: Dual 12’ full circulating bars - Nozzles: spaced on 12” centers across both spray bars - Spray bar shift: 18” each way (contingent on model and interferences) - Polymer bar nozzle: Based on application rates - Rejuvenator bar nozzle: Based on application rates - Controller: - Plus+1 CAN Bus control and onboard diagnostic system - 7” high-resolution, color graphic display - Controller for automatic application rate control - Automatic valves for cab control of distributor functions - Settings for flow calibration, speed calibration, and application rates - Capable of storing up to 12 preset application rates for instant retrieval - Readouts include FPM, GPM, total feet sprayed, total gallons sprayed, total square yards sprayed, flow and speed calibration, application rate, tank temperature, hour meter and spray bar width - Controller provides accurate applications regardless of changes in speed or spray bar width during operations - Includes radar horn ground speed sensor - Convenient external controls mounted on rear fender - Hydrostatic Power: - Hydrostatic, PTO driven tandem pump - The hydrostatic tandem pump is a variable displacement axial piston pump - Hydraulic reservoir: Tank with temperature sight gauge and level indicator - Additional Standard Equipment: - Back-up alarm - Mud flaps - Beacon light - Filter basket strainers for front and rear tank - Hose hanger brackets - Optional Equipment: - Front tank hand spray wand with rubber hose - Rear tank hand spray wand with rubber hose - Frame Mounted Toolbox - Extra Fender Mounted Toolbox - LED Work Light Package: Qty. 4 - Back-up/ reverse camera w/ in cab color display - Full Width Mud Flap - Training, Support and Company - Training and support - Optional on-site operator training by Dealer - Two-year, unlimited hours warranty against any manufacturer defects - Extended warranty available - 60+ years manufacturing experience - ISO 9001 Certified - Sourcewell contract # 060122-VTL - Manufactured in the USA