Product Specifications

LeeBoy TD600 Tack Distributor

The following specifications are based on the TD600 Tack Distributor manufactured by LeeBoy.  Because of continuous equipment research and development, we reserve the right to change these specifications. For more information, questions or assistance preparing specifications, please contact your local LeeBoy Dealer.

*Published Weights are approximate weight and will vary contingent on options.

- Dimensions: - Length: 17’5” - Width: 7’8” - Height: 6’9” - Tank: 600 gallons - Trailer Specifications: - 6,000 lb. axles - 5,000 lb. jackleg - 3” pintle hitch with 10” of vertical adjustment - Wired for towing with 7-pin connector - Tires: ST 225/75D15 - Electric brakes on axles - Safety chains - Emergency breakaway switch with battery backup - Shipping weight: 5,660 lbs. - Loaded weight: 10,750 lbs. - Tank Dimensions: - Length: 64” - Width: 70” - Height: 42” - Engine: - Kubota diesel – 24.8 hp (18.5 kW) electric start - Fuel: 16 gallons - Asphalt Pump: - Specially designed Viking 150 GPM pump - Heat System: - One 300,000 BTU Diesel burner - Fuel: 16 gallons (pulls from engine fuel tank) - Material temperature set point adjustable from in-cab controls automatically turns off burner when reached. - Spray Bar: - 14’ Spray Bar - Outer 3’ sections of spray bar fold vertically. - Spray nozzles spaced every 4” along bar. - Levers for individual spray valves can be flipped over to deactivate and shorten spray width as needed. - Wing sections contain angle switch to automatically disable spray when folded up. - Wireless In-Cab Controls: - Color display with readouts for: engine rpm, engine fault status, system faults, engine hours, ground speed, application rate, spray width, material temperature, material temperature set point, mode selection, asphalt pump current speed, valve status, totals screen - Auto/Manual pump selector - Manual pump speed dial - Engine start/stop - Bar circulate/off/spray switch - Bar section on/off for left wing, left center, right center, right wing - Mode selector: off, spray/bar circulate, tank circulate/hand spray, reverse suction, cleanout, calibration - E-stop - Temperature Gauge: - Analog Dial 0 - 500⁰ F - Adjustable angle faceplate - Flue Design - Five pass flue system - Galvanized exhaust stack - Manhole: - 20” diameter - 10” diameter fill hose - 3 psi pressure relief - Additional Standard Equipment - Waste bucket mount - 2” tank overflow - Flush tank: 7-gallon tank - Clean out: 4” - Sampling valve: 1” - LED work light package: includes 2 work lights - Beacon light with mount - Optional Equipment: - Diesel spray down system (pulls from engine fuel tank): includes electric pump, retracting hose reel, and spray gun - Overnight Heat 120 volt/1500-watt Immersion element - Frame mounted toolbox - Wheel chocks - Fire extinguisher: 5lbs - Hand Spray Wand - 5’ 2” wand with cold handles - On/off valve - Removable spray tip - Hose and reel: .5” x 40’ heavy-duty recoil hose and reel - Overnight battery charger / tender - Training, Support and Company - Training and support manuals - Optional on-site operator training by Dealer - Two-year, unlimited hours warranty against any manufacturer defects - Extended warranty available - 60+ years manufacturing experience - ISO 9001 Certified - Sourcewell Contract #060122-VTL - Manufactured in USA